Exercise 1: Introduction to ANSYS Workbench ver. 19

Introduction

Figure 1 shows a flow chart where it is possible to observe the general steps to generate a simulation using any CFD software package. ANSYS Workbench assists the user in carrying out the tasks involved in an analysis process; typically the software is divided in three main parts:

- A pre-processor.

- A Solver.

- A post-processor.

Figure N°1. The Analysis Process.

Exercise number 1 consists in a pre-processor task, and it is necessary to take into account the steps marked in red square in the figure 1.

Purpose

Get familiar with selected components of ANSYS Workbench: ANSYS DesignModeler (geometry definition) and ANSYS Meshing (mesh generator) and ANSYS Fluent (flow solver).

Project start-up

Before starting a solution of any flow problem using the ANSYS Fluent we will need to create geometry and make the computational mesh. Next the Fluent solver has to be properly set-up. The numerical results can be later analysed using either the Fluent or the external post processing tool. This is called the data post processing. During the labs we will use also Fluent for flow simulation and for data analysis.

Drag and drop a Fluid Flow (Fluent) component from Analysis Systems into the Project Schematic (white window).  As result, all necessary components will be shown altogether.

Figure N°2. ANSYS Workbench

Alternatively, each component can be shifted separately from Component Systems window into the Project Schematic window. We could start by adding the Geometry box, later Mesh and finally the Fluent box, as shown below.

Figure N°3. ANSYS Workbench

Symbol  means that this compoment is undefined yet. The symbol  changes to  if the compoment problem has been properly specified/updated. Each project component in the Project Schematic has to be properly defined before gooing to the next one. The meaning of some symbols is given below:

- Problem is undefined. User has to introduce some changes into the given project component.

- - Allows to update the data so they can be used in the remaining part(s) of the project loop.  (Update Project)

- Project refreshing is required. (Refresh Project)

Geometry

Start the ANSYS Workbench. Drag and drop Geometry box into the Project Schematic. Right click on the Geometry cell to select and run DesignModeler.

Figure N°4. ANSYS Workbench – Geometry Design Modeler

Select Centimetres as a desired unit length in Units.

Figure N°4. ANSYS Workbench – Geometry DesignModeler

In the Tree Outline (on the left) select the XYPlane and click on the Sketching button. The Sketch1 will pop-up automatically when we start our drawings.

Click on the Z axis (the triad in the bottom right in Graphics window) to position the drawing window in the X-Y plane. We are going to create a following sketch (Note that the fluid flow will be from left to right):

h

48 cm

70 cm

2 cm

3 cm

Figure N°5. Exercise N°1 Geometry

Select the Draw/Rectangle toolbox. Sketch the arbitrary rectangular in the Graphics window (it will be used for creating right side of our geometry – outflow section).

Change to Constraints/Coincident under the Sketching Toolboxes and make a link between the left bottom corner of the rectangle and the origin of the coordinate system (select rectangle’s corner and respectively X and Y axis). The left bottom corner of the rectangle will be shifted/make coincident with the origin of the coordinate system.  Activate Show Constraints (specify YES) under Details of Sketch to verify all constraints.

Specify dimensions of the rectangle. Select Dimensions\General. Select the vertical side of the rectangle and specify a proper size (Figure N°5) under Details View\Dimensions. Do the same for the horizontal side. The blue colour means that the dimensions/constaints have been correctly specified.

Switch to Modeling. Create the second sketch (smaller rectangle to the left). Select XYPlane and add the Sketch symbol as shown below.

Make sure that the proper sketch is selected when working on new geometry components.

Figure N°6. ANSYS Workbench – Geometry DesignModeler

Create the second rectangular (inlet section), specify dimensions and constaints. Right upper corner of the new rectangular has to be linked with the left upper corner of the previously generated rectangular (Constraints\Coincident). See figure below. Specify dimensions of the new rectangle. Select Dimensions\General. Select the vertical side of the rectangle and specify a proper size (Figure N°5) under Details View\Dimensions

Figure N°7. Exercise N°1 Geometry Design

Use these two sketches for creating surfaces Concept/Surfaces from Sketches. Click on a newly generated SurfaceSk under Geometry. Hold the Ctrl button and select both sketches from the tree, confirm their selection by clicking Apply next to Base Objects cell. Right click and push on Generate to make new surfaces.

Change the properties of the newly created bodies from Solid to Fluid (Part, Body: Surface Body) under Details of Surface Body/ Fluid/Solid.

Note that any surface can be hiden in order to allow an easier manipulation on the remaining componets of the geometry (select surface body, right click and use Hide Body). One can completely deactivate the given surface so it will not be used in the process of mesh genearation (select surface body, right click and use Suppress Body to see how it affects the geometry).

The present object consists of two Parts. This means that both parts are seen as two separate elements and that there is no link between them (it means that physically there will be no flow from one part to the other). One part has to be made to enforce the continuity. Select both surfaces (holding shift button) under 2 Parts, 2 Surfaces right click and select Form New Part. Check whether both surfaces are defined as Fluid.

One can verify coordinates (or dimension) of selected components (points, lines, surfaces etc). Go to Tools/Analysis Tools/Entity Information and select point selection filter (see below). Select arbitrary point in the Graphics window and verify the coordinates (in the exercise you will need to know the coordinates of inlet, so write them down somewhere).

Figure N°8. ANSYS Workbench – Geometry DesignModeler

Save the project and close DesignModeler.

Mesh

Drag and drop the Mesh cell into the Geometry box in the Project Schematic. Double click on the Mesh to open the meshing program. Select the Mesh under the Outline.

In Details specify the following under Sizing:

- Size Function: Adaptive

- Element Size: type 5 mm instead of Default (pay attention to the selected unit)

Attention: In Workbench the decimal mark (symbol separating the integral from the fractional part of the real number) is comma „ , ”. In Fluent the decimal mark is dot “ . ”.

Click on Generate Mesh to make the first mesh.

Now we are going to make a finer mesh. This requires a specification of a proper size of the cell elements on selected edges. Activate the edge selection filter option in the upper menu panel. Select all vertical edges (use Ctrl to select multiple edges) right clik on the Mesh in the Outline panel and select Insert/Sizing. Provide the proper Size of the cell elements (on a 30 mm edge we expect to have 10 divisions) or simply provide the Number of divisions on selected edge.

The specification from the edges has to be transformed to the other edges and to the interior of the domain. Select both surfaces (first activate surface selection filter), right click and insert Insert/Face Meshing. Make sure, that Definition/Mapped mesh  is turned on. Generate the grid.

Figure N°9. ANSYS Workbench – Meshing

Insert the specific names at the edges corresponding to the inlet and outlet. Select Geometry under the Object window. Switch selection mode to edges. Select the leftmost vertical line (Graphics window), right click and select Create Named Selection. Specify the name “velocity_inlet”. Specify the edge name “pressure_outlet” at the outlet from the domain (rightmost vertical edge). Specify the “Wall_bottom” name on the bottom horizontal line after the step.  The default boundary condition on the remaining (unnamed) edges will be wall.

Export the mesh using File/Export (select file extension as Fluent Input Files: *.msh).

Open Fluent program (Menu Start/ANSYS 19.0/Fluent 19.0). Set: 2D, serial, double precision, specify the working directory. Run Fluent. Read the mesh (*.msh file), File/Read/Mesh. Select option Display Mesh After Reading.

Verify a correctness of the boundary conditions specification (inlet-blue colour, outlet-red, walls-white). Save the *.cas file (File/Write/Case) and close Fluent.

We go back to Workbench. Drag and drop the Mesh cell into the Geometry box in the Project Schematic. We will employ an automatic mesh generation technique now.

Specify the following Mesh settings.

Defaults:

Physics Preference: CFD

Sizing:

Use Advanced Size Function: On Proximity and Curvature

Generate the new mesh. Change the settings under Physics Preference and verify their effect on quality of generated mesh.

One can introduce the boundary layer mesh near to walls. The aim is to make the grid denser in the high velocity gradient flow regions. Right click on Mesh cell and introduce  Insert/Inflation. Activate the Face symbol in the geometry selection filter in upper menu panel and select all surfaces under Geometry selection cell. Select all edges corresponding to walls under Boundary. Use the Ctrl button to select multiple edges. Set the number of layers to 10 (Maximum Layers) and generate the mesh.

Figure N°10. ANSYS Workbench – Meshing

Save the Project.

Additional problem

Make the geometry shown below and generate the mesh

Figure N°11. ANSYS Workbench – Additional Project